Working Note

Methods of Accuracy Assessment in the SIBERIA project

Heiko Balzter, Institute of Terrestrial Ecology

21/6/99

# Purpose of Accuracy assessment

- To compare the performance of different classification methodologies

- To quantify the reliability of a thematic map product (quality checking)

# Technical aspects

## Methods of accuracy assessment

A literature search on accuracy assessment was performed at ITE to get an overview of available techniques and statistical methods.

The summary of the results of an accuracy assessment are the accuracy report. Each map product should be accompanied by an accuracy report. Minimum requirements for an accuracy report are to provide the user with information about

	geometric accuracy: standard deviation in x and y direction during co-registration;

	classification accuracy: coefficient of agreement (CA) for training areas to assess their quality;

	map accuracy: co-occurence matrix, class-specific errors of commission and of omission, overall accuracy and CA between classification and ground data.

An a priori coefficient of agreement for nominal data is  (tau). It estimates the expected chance agreement from a priori knowledge about the expected class frequencies. If nothing is known a priori, each class is assumed to occur with equal probabilities.

An a posteriori coefficient of agreement is  (kappa). It estimates the expected chance agreement from the observed marginal distributions of the co-occurrence matrix.

Because the assumption of equal probabilities for all land cover classes is not justified in the case of the SIBERIA project (we know that there is more forest than water for example),  is recommended for use.

Some image processing software packages (like PCI) offer accuracy assessment methods. However, the implementation of these methods is not flexible enough in some cases. To implement an accuracy assessment, a C programme (KAPPA.C) has been written to estimate , , their 95% confidence intervals, standard deviations and significance probabilities, and to print a co-occurence matrix between two 8 bit images. Another C programme (COMPARE2KAPPAS.C) has been completed to compare two coefficients of agreement and test the statistical significance of them being different. Both programmes can be downloaded from the password protected FTP server at Swansea.

## A supervised classification at Ust-Ilimsk

The classification discussed below was the first we attempted to do. It is simplistic and serves for demonstration purposes of methods of accuracy assessment only.

### Assessment of geometric accuracy

We have initially been working on the Ust-Ilimsk test site. All images and GIS layers were co-registered to ERS_32371_2421 (GEC product) using around ten ground control points. The intersection between GIS, ERS_32371_2421 and JERS_28593_201/202 is covering approximately 2800 km2. The following database in UTM geometry has been set up (Table 1). GIS variables represent the year 1991, and were recently updated to the requirements of the customers.

Table 1: Structure of the database of SAR images and GIS layers.

| Database channel | Orbit and frame number |
| --- | --- |
| amplitude ERS-1 23/09/1997 | 32371_2421 |
| amplitude ERS-2 24/09/1997 | 32371_2421 |
| amplitude ERS-2 27/05/1998 | 32371_2421 |
| 1 day coherence 23-24/09/1997 | 32371_2421 |
| amplitude JERS-1 04/05/1997 | 28593_201, 28593_202 |
| amplitude JERS-1 31/07/1997 | 29911_202 |
| forest type (coniferous, mixedwood, deciduous) |  |
| relative stocking (2 classes) |  |
| forest age (2 classes) |  |
| total growing stock per hectare (6 classes) |  |

The standard deviation of the coordinate transformations in x- and y-direction varied between 21 m and 34 m, which is less than the pixel spacing of 50 m. Nearest neighbour resampling was used.

### Assessment of classification accuracy

A first supervised maximum likelihood classification (classes: forest, non-forest, river) was undertaken using all satellite channels except the second JERS pass. The highest information content was found to be in L-band amplitude of JERS-1 and the C-band ERS coherence. The signature separability was measured by the Bhattacharrya distance (scaled to 0  B  2), and was greater than 1.94 for each pair of classes (very good separability). The overall accuracy of the training areas was 97.3% and =0.95.

### Assessment of map accuracy

The overall accuracy of the classification was 71.5%,  = 0.75, s = 0.00057. However, from the co-occurence matrix between the classification and the GIS (Table 2), the reason for this low overall accuracy lies in the low consumer's accuracy of non-forest (14.01%). A high proportion of forest classes in the GIS have been classified as non-forest. As the time lag between the GIS and the remote sensing data acquisition is 6-7 years, it is highly likely that new clearcuts have been made. The effects of misclassification and land use change combine in this co-occurence matrix and make it inadequate for an assessment of classification accuracy.

Table 2: Co-occurence matrix of pixels in the classified image and the GIS. Pixel spacing is 50 m.

| reference class classified as | forest | non-forest |  | consumer's accuracy |
| --- | --- | --- | --- | --- |
| forest | 749 752 | 14 336 | 764 088 | 98.12% |
| non-forest | 305 060 | 49 684 | 354 744 | 14.01% |
|  | 1 054 812 | 64 020 | 1 118 832 |  |
| producer's accuracy | 71.08% | 77.61% |  |  |

# Open questions

- How does the use of GEC or GTC products influence the accuracy of the produced map?

- How do we implement a methodology for accuracy assessment for a map given a time lag with the ground data of several years?

- How can we assess systematically which classification methodology yields the highest accuracy on several / all test sites?

- How do we assess the accuracy of the global map, given the restricted availability of ground data only for the test sites?

# References

Aronoff, S., 1982, The Map Accuracy Report: A User's View: Photogrammetric Engineering and Remote Sensing, v. 48, p. 1309-1312.

Aronoff, S., 1985, The Minimum Accuracy Value as an Index of Classification Accuracy: Photogrammetric Engineering and Remote Sensing, v. 51, p. 99-111.

Congalton, R.G., 1988, A Comparison of Sampling Schemes Used in Generating Error Matrices for Assessing the Accuracy of Maps Generated from Remotely Sensed Data: Photogrammetric Engineering and Remote Sensing, v. 54, p. 593-600.

Congalton, R.G., 1991, A Review of Assessing the Accuracy of Classifications of Remotely Sensed Data: Remote Sensing of Environment, v. 37, p. 35-46.

Congalton, R.G., and Green, K., 1993, A Practical Look At the Sources of Confusion in Error Matrix Generation: Photogrammetric Engineering and Remote Sensing, v. 59, p. 641-644.

Fernandez, R.N., Lozanogarcia, D.F., Deeds, G., and Johannsen, C.J., 1991, Accuracy Assessment of Map Coordinate Retrieval: Photogrammetric Engineering and Remote Sensing, v. 57, p. 1447-1452.

Fitzgerald, R.W.a.L., B.G., 1994, Assessing the Classification Accuracy of Multisource Remote Sensing Data: Remote Sensing of Environment, v. 47, p. 362-368.

Foody, G.M., 1992, On the Compensation For Chance Agreement in Image Classification Accuracy Assessment: Photogrammetric Engineering and Remote Sensing, v. 58, p. 1459-1460.

Ginevan, N.E., 1979, Testing Land-Use Map Accuracy: Another Look: Photogrammetric Engineering and Remote Sensing, v. 45, p. 1371-1377.

Gopal, S., and Woodcock, C., 1994, Theory and Methods For Accuracy Assessment of Thematic Maps Using Fuzzy-Sets: Photogrammetric Engineering and Remote Sensing, v. 60, p. 181-188.

Janssen, L.L.F., and Vanderwel, F.J.M., 1994, Accuracy Assessment of Satellite-Derived Land-Cover Data - a Review: Photogrammetric Engineering and Remote Sensing, v. 60, p. 419-426.

Martin, L.R.G., and Howarth, P.J., 1989, Change-Detection Accuracy Assessment Using Spot Multispectral Imagery of the Rural Urban Fringe: Remote Sensing of Environment, v. 30, p. 55-66.

Mowrer, H.T., Czaplewski, R.L., Hamre, R.H., Rocky Mountain Forest and Range Experiment Station (Fort Collins, C., International, S.o.S.A.A.i.N.R., Environmental, S., and Fort Collins, C., 1996, Spatial accuracy assessment in natural resources and environmental sciences : second international symposium: Fort Collins Colo. (3825 East Mulberry St. Fort Collins 80524-8597), U.S. Dept. of Agriculture Forest Service Rocky Mountain Forest and Range Experiment Station, 728 p.

Muller, S.V., Walker, D.A., Nelson, F.E., Auerbach, N.A., Bockheim, J.G., Guyer, S., and Sherba, D., 1998, Accuracy assessment of a land-cover map of the Kuparuk River Basin, Alaska: Considerations for remote regions: Photogrammetric Engineering and Remote Sensing, v. 64, p. 619-628.

Naesset, E., 1996, Conditional Tau-Coefficient For Assessment of Producers Accuracy of Classified Remotely-Sensed Data: Isprs Journal of Photogrammetry and Remote Sensing, v. 51, p. 91-98.

Skidmore, A.K., and Turner, B.J., 1992, Map Accuracy Assessment Using Line Intersect Sampling: Photogrammetric Engineering and Remote Sensing, v. 58, p. 1453-1457.

Stehman, S.V., 1992, Comparison of Systematic and Random Sampling For Estimating the Accuracy of Maps Generated From Remotely Sensed Data: Photogrammetric Engineering and Remote Sensing, v. 58, p. 1343-1350.

Stehman, S.V., 1995, Thematic Map Accuracy Assessment From the Perspective of Finite Population-Sampling: International Journal of Remote Sensing, v. 16, p. 589-593.

Stehman, S.V., 1996a, Estimating the Kappa-Coefficient and Its Variance Under Stratified Random Sampling: Photogrammetric Engineering and Remote Sensing, v. 62, p. 401-407.

Stehman, S.V., 1996b, Use of Auxiliary Data to Improve the Precision of Estimators of Thematic Map Accuracy: Remote Sensing of Environment, v. 58, p. 169-176.

Stehman, S.V., 1997a, Estimating Standard Errors of Accuracy Asessment Statistics under Cluster Sampling: Remote Sensing of Environment, v. 60, p. 258-269.

Stehman, S.V., 1997b, Selecting and interpreting measures of thematic classification accuracy: Remote Sensing of Environment, v. 62, p. 77-89.

Stehman, S.V., and Czaplewski, R.L., 1998, Design and analysis for thematic map accuracy assessment: Fundamental principles: Remote Sensing of Environment, v. 64, p. 331-344.

Story, M.a.C., R.G., 1986, Accuracy Assessment: A User's Perspective: Photogrammetric Engineering and Remote Sensing, v. 52, p. 397-399.

Verbyla, D.L., and Hammond, T.O., 1995, Conservative Bias in Classification Accuracy Assessment Due to Pixel- By-Pixel Comparison of Classified Images With Reference Grids: International Journal of Remote Sensing, v. 16, p. 581-587.