DATA FILE DEFINITION

1. Data Files

1.1. Slant range datasets

| Name | Format | Bit Depth | Resolution | Description |
| --- | --- | --- | --- | --- |
| ’E1-orbitno’_’frameno’.wbb | SUN RASTER | 24 | 100 m | combination of ampl1.ras, ampl2.ras and coher.ras (slant range) |
| ampl1.ras | SUN RASTER | 16 | 50 m | ERS-1 amplitude, tandem data (slant range) |
| ampl2.ras | SUN RASTER | 16 | 50 m | ERS-2 amplitude, tandem data (slant range) |
| ampl3.ras | SUN RASTER | 16 | 50 m | ERS-1/2 amplitude, spring coverage (slant range) |
| coher.ras | SUN RASTER | 8 | 50 m | coherence map (slant range) |
| ampl1_dmp | ASCII |  |  | parameter file for ampl1.ras |
| ampl2_dmp | ASCII |  |  | parameter file for ampl2.ras |
| ampl3_dmp | ASCII |  |  | parameter file for ampl3.ras |

1.2. Geocoded datasets

| Name | Format | Bit Depth | Resolution | Description |
| --- | --- | --- | --- | --- |
| geo_ampl1.ras | SUN RASTER | 16 | 50 m | ERS-1 amplitude, tandem data |
| geo_ampl2.ras | SUN RASTER | 16 | 50 m | ERS-2 amplitude, tandem data |
| geo_ampl3.ras | SUN RASTER | 16 | 50 m | ERS-1/2 amplitude, spring coverage |
| geo_coher.ras | SUN RASTER | 8 | 50 m | coherence map |
| geo_dem.ras | SUN RASTER | 16 | 50 m | digital elevation model (only in case of GTC) |
| geo_slope.ras | SUN RASTER | 8 | 50 m | slope map (only in case of GTC) |
| geo_ampl1_dmp | ASCII |  |  | parameter file for geo_ampl1.ras |
| geo_ampl2_dmp | ASCII |  |  | parameter file for geo_ampl2.ras |
| geo_ampl3_dmp | ASCII |  |  | parameter file for geo_ampl3.ras |

In case of ellipsoid corrected products (GECs) the datasets are geocoded to a mean height of 700 m. This is the mean height all over the SIBERIA project region.

2. Parameter file definition

2.1. General parameters

| Name | Format | Units | Description | Example |
| --- | --- | --- | --- | --- |
| C_ELL_ID | A4 | - | ellipsoid code | WG84 |
| C_ELL_MAX | E | [km] | semi-major axis of ellipsoid | 6378.1440000 |
| C_ELL_MIN | E | [km] | semi-minor axis of ellipsoid | 6356.7590000 |
| C_SITE_NAME | A | - | name of scene | Bratsk |
| C_SHIFT_ID | A4 | - | datum shift code - for geocoding output | WG84 |
| F_GCP_NAM | A40 | - | DFD internal filename | /home/horex04/siberia/insar/gemos.gcp |
| F_SAR_COL | I |  | number of colums in raster file | 2434 |
| F_SAR_LIN | I |  | number of lines in raster file | 2600 |
| F_WINDOW_NAM | A40 | - | DFD internal filename | /home/horex04/siberia/insar/geo_dem.ras |
| G_AZIMUTH_LOOKS | F |  | number of looks in azimuth | 10.0000000 |
| G_INANG_CENT | E | [deg] | incidence angle at mid range | 23.2330000 |
| G_INANG_RGF | E | [deg] | incidence angle at far range | 26.4288899 |
| G_INANG_RGN | E | [deg] | incidence angle at near range | 19.4569800 |
| G_ORB_ADF | A1 | - | ascending / descending orbit flag: ’A’/’D’ | D |
| G_RANGE_LOOKS | F |  | number of looks in range | 2.0000000 |
| G_RANGE_RGN | E | [km] | range near distance | 835.4733138 |
| G_SENS_EDIR | E |  | sensor track direction, scene centre | 198.3745000 |
| I_EFF_BASELINE | F | [m] | effective baseline | 233.9083890 |
| I_HEIGHT_INTERVAL | F | [m] | height interval | 39.6546630 |
| I_STATE_VECTOR_X0 | F | [m] | state vector in X | -1174480.7078379 |
| I_STATE_VECTOR_X1 | F | [m] | state vector in X | -1172933.8370202 |
| I_STATE_VECTOR_X2 | F | [m] | state vector in X | -1171348.5062057 |
| I_STATE_VECTOR_X3 | F | [m] | state vector in X | -1169724.7853943 |
| I_STATE_VECTOR_X4 | F | [m] | state vector in X | -1168062.7645861 |
| I_STATE_VECTOR_Y0 | F | [m] | state vector in Y | 3585180.9758982 |
| I_STATE_VECTOR_Y1 | F | [m] | state vector in Y | 3612340.9440184 |
| I_STATE_VECTOR_Y2 | F | [m] | state vector in Y | 3639432.3021287 |
| I_STATE_VECTOR_Y3 | F | [m] | state vector in Y | 3666454.5102289 |
| I_STATE_VECTOR_Y4 | F | [m] | state vector in Y | 3693407.0483192 |
| I_STATE_VECTOR_Z0 | F | [m] | state vector in Z | 6080734.6523465 |
| I_STATE_VECTOR_Z1 | F | [m] | state vector in Z | 6064973.4502537 |
| I_STATE_VECTOR_Z2 | F | [m] | state vector in Z | 6049097.7881688 |
| I_STATE_VECTOR_Z3 | F | [m] | state vector in Z | 6033107.9660916 |
| I_STATE_VECTOR_Z4 | F | [m] | state vector in Z | 6017004.2840224 |
| I_STATE_VECTOR_T0 | F | [sec] | UTC time for state vector | 15215.4549999 |
| I_STATE_VECTOR_T1 | F | [sec] | UTC time for state vector | 15219.6219999 |
| I_STATE_VECTOR_T2 | F | [sec] | UTC time for state vector | 15223.7890000 |
| I_STATE_VECTOR_T3 | F | [sec] | UTC time for state vector | 15227.9560000 |
| I_STATE_VECTOR_T4 | F | [sec] | UTC time for state vector | 15232.1229999 |
| MC_VERS_SW | A | - | actual SW version of geocoding software | 2.1 |
| R_AQUI_DATE | A8 |  | data acquisition date [YYYYMMDD] | 19970923 |
| R_AQUI_TIME | A6 |  | data acquisition time [hhmmss] | 041335 |
| R_FRAME_ID | I |  | frame number | 2421 |
| R_ORBIT_NO | I |  | orbit number | 32371 |
| R_SAT_ID | A2 |  | satellite ID | E2 |
| R_PRO_TYPE | A |  | product type | GTC |
| R_SFACILITY_ID | A |  | source product facility identifier | D-PAF |
| R_SLA_LLLA | F | [deg] | geographic latitude of scene lower left corner (of coregistered Scene, slant range) | 58.2695500 |
| R_SLA_LLLO | F | [deg] | geographic longitude of scene lower left corner (of coregistered Scene, slant range) | 101.0548000 |
| R_SLA_LRLA | F | [deg] | geographic latitude of scene lower right corner (of coregistered Scene, slant range) | 58.0563199 |
| R_SLA_LRLO | F | [deg] | geographic longitude of scene lower right corner (of coregistered Scene, slant range) | 103.5778999 |
| R_SLA_SCLA | F | [deg] | geographic latitude of scene centre (of the original SLCI) | 58.6169999 |
| R_SLA_SCLO | F | [deg] | geographic longitude of scene centre ( of the original SLCI) | 102.9270000 |
| R_SLA_ULLA | F | [deg] | geographic latitude of scene upper left corner (of coregistered Scene, slant range) | 59.1689599 |
| R_SLA_ULLO | F | [deg] | geographic longitude of scene upper left corner (of coregistered Scene, slant range) | 102.3725000 |
| R_SLA_URLA | F | [deg] | geographic latitude of scene upper right corner (of coregistered Scene, slant range) | 58.9521000 |
| R_SLA_URLO | F | [deg] | geographic longitude of scene upper right corner (of coregistered Scene, slant range) | 104.0387000 |
| R_SPRO_TYPE | A |  | source product type | SLCI |
| V_CAL_CONST | F |  | calibration constant | 65026.0000000 |
| V_PROD_DATE | A8 |  | UTC when product was generated [YYYYMMDD] | 19980717 |
| V_REPP_IMAGE | F |  | replica pulse power variation | 197499.0000000 |

2.2. Parameters only for geocoded products

| Name | Format | Units | Description | Example |
| --- | --- | --- | --- | --- |
| C_COUNTRY_ID | A4 | - | country code (ESA = UTM) | ESA |
| C_ZONE_ID | A4 | - | UTM zone | UT48 |
| C_PRO_NAME | A | - | projection name (Transverse Mercator) | TM |
| R_FRA_CEEA | F | [m] | cartographic easting of east border | 442425.0000000 |
| R_FRA_CNNO | F | [m] | cartographic northing of north border | 6562275.0000000 |
| R_FRA_CSNO | F | [m] | cartographic northing of south border | 6436525.0000000 |
| R_FRA_CWEA | F | [m] | cartographic easting of west border | 319975.0000000 |